C***********************************************************************

!  This component has been assigned Type Number 205. If that number conflicts with

!  another user Type number, you will need to change it and recompile the appropriate

!  dll.

SUBROUTINE TYPE205(TIME,XIN,OUT,T,DTDT,PAR,INFO,ICNTRL,*)

!DEC$ATTRIBUTES DLLEXPORT :: TYPE205

C ----------------------------------------------------------------------

C

C       TYPE 205 : CONUIT MODEL (PIPE or DUCT)

C

C   FLOW:   kg/s            (Fluid mass flow rate)

C   TIN:    C               (Fluid inlet temperature)

C   TAMB:   C               (Ambient temperature)

C   P2:     kPa             (Pressure at conduit outlet)

C   HIA:    kW/C            (Heat transfer coeff*area, inside surface)

C   HOA:    kW/C            (Heat transfer coeff*area, outside surface)

C   CM:     kJ/C            (Thermal capacitance of conduit material)

C   VOL:    m3              (Volume of conduit)

C   K:      0.001/(kg-m)    (Flow Resistance)

C   H:      m               (height of outlet relative to inlet)

C                  (sign convention: positive H --> uphill flow)

C

C  The last parameter, MODE, has the following meaning:

C   MODE = -2 => water, detailed dynamics (one diff.eq.)

C   MODE = -1 =>   air, detailed dynamics (one diff.eq.)

C   MODE =  1 =>   air,   simple dynamics (no  diff.eq.)

C   MODE =  2 => water,   simple dynamics (no  diff.eq.)

C

C***********************************************************************

DOUBLE PRECISION   XIN,OUT

REAL      PAR,S,K

INTEGER   INFO

DIMENSION  XIN(5),OUT(2),PAR(7),INFO(15)

C     REAL      PAR,SAVED,K

C     LOGICAL   FRZTO

C     INTEGER   ITIME,IOSTAT

C     DIMENSION  XIN(5),OUT(2),PAR(7),SAVED(11)

C     DIMENSION  IOSTAT(5)

C*    COMMON    /PROPER/ RHOA,RHOW,CPA,CPW

C*    COMMON    /CHRONO/ TIME,TSTEP,TTIME,TMIN,ITIME

C*    COMMON    /SOSCOM/ RTOLX,ATOLX,XTOL

C*    COMMON    /XINIT/  INIT,NSAVED

C  The RHOA,RHOW,CPA,CPW properties @300 K, 1 atm

C  The units are:

C      RHOA,RHOW - kg/m^3

C      CPA,CPW   - kJ/kg-K

COMMON   /SIM/ TIME0,TFINAL,DELT,IWARN

COMMON  /STORE/ NSTORE,IAV,S(5000)

DATA  RHOA,RHOW,CPA,CPW /1.1614,999,1.007,4.179/

DATA  INIT /0/

DATA GRAV /9.80665/

!    Set the version information for TRNSYS

IF (INFO(7).EQ.-2) THEN

INFO(12) = 16

RETURN 1

ENDIF

IF (INFO(7) .EQ. -1) THEN

INFO(6)=2

INFO(9)=1

INFO(10)=11

CALL SETSTORAGESIZE(11,INFO(15))

CALL TYPECK(1,INFO,5,7,0)

CALL RCHECK

RETURN 1

ENDIF

ISAV=INFO(10)

FLOW=  XIN(1)

P2=    XIN(2)

TIN=   XIN(3)

TAMB=  XIN(4)

TOUT=  XIN(5)

C     FRZTO=((IOSTAT(1).LT.-1).OR.(IOSTAT(1).EQ.2)) .AND.

C    & ((IOSTAT(3).LT.-1).OR.(IOSTAT(3).EQ.2))

HIA=   PAR(1)

HOA=   PAR(2)

CM=    PAR(3)

VOL=   PAR(4)

K=     PAR(5)

H=     PAR(6)

MODE=  PAR(7)

IF (ABS(MODE) .EQ. 1) THEN

CP=CPA

RHO=RHOA

IF (TIN.LE.-30.) TIN=-30.               ! added 2/24/94

IF (TIN.GT.65.)  TIN=65.

ELSE

CP=CPW

RHO=RHOW

IF (TIN.LE.0.)   TIN=0.

IF (TIN.GT.120.) TIN=120.

ENDIF

IF ((HIA.GT.0.).AND.(HOA.GT.0.))  THEN

UA=(HIA*HOA)/(HIA+HOA)

ELSE

UA=0.

ENDIF

C*    IF (ITIME .EQ.1) THEN

IF (INFO(7).EQ.-1) THEN

IF (INIT.EQ.0) THEN

C           SAVED(9)=0.

C           SAVED(11)=TIN

S(ISAV+8)=0.

S(ISAV+10)=TIN

ENDIF

C        IF (SAVED(9).GT.TIME) THEN

C           SAVED(9)=0.

IF (S(ISAV+8) .GT. TIME) THEN

S(ISAV+8)=0.

ENDIF

ENDIF

C     IF (TIME.GT.SAVED(9)) SAVED(10)=SAVED(11)

C     TOLD=SAVED(10)

IF (TIME .GT. S(ISAV+8)) S(ISAV+9)=S(ISAV+10)

TOLD=S(ISAV+9)

P1=P2+(RHO*GRAV*H*0.001)+K*FLOW*ABS(FLOW)

C---multiplication by 0.001 converts from Pa to kPa.

IF (FLOW .LE. 0.00001) THEN

IF (ABS(MODE).EQ.MODE)THEN

OUT(1)=TIN

C           SAVED(11)=TOLD

S(ISAV+10)=TOLD

ELSE

OUT(1)=0.

C           SAVED(11)=TIN

S(ISAV+10)=TIN

ENDIF

OUT(2)=P1

C        IOSTAT(1)=1

C        IOSTAT(2)=1

RETURN	1

ENDIF

100   TSS=TAMB

A=UA/(FLOW*CP)

DT=TIN-TAMB

IF (A.LE.20. .AND. ABS(DT).GE.1.E-10) THEN

TSS=TAMB+DT*EXP(-A)

ENDIF

TAUX=RHO*VOL/FLOW

IF (ABS(MODE).EQ.MODE) THEN

C  simplified dynamics (no differential equations)

250      T=TSS

IF ((CM.GT.0.) .AND. (HIA.GT.0.)) THEN

C*          A=FLOW*CP*TSTEP*(1.+HOA/HIA)/CM

A=FLOW*CP*DELT*(1.+HOA/HIA)/CM

DT=TOLD-TSS

IF (A.LE.20. .AND. ABS(DT).GE.1.E-10) THEN

T=TSS+DT*EXP(-A)

ENDIF

ENDIF

C        TOUT=DELAY(T,TAUX,SAVED(1),SAVED(7),SAVED(8),SAVED(9))

TOUT=DELAY(T,TAUX,S(ISAV),S(ISAV+6),S(ISAV+7),S(ISAV+8),INFO)

OUT(1)=TOUT

C        SAVED(11)=T

S(ISAV+10)=T

ELSE

C  detailed dynamics (one differential equation)

C*       DTINDT=(TIN-TOLD)/TSTEP

DTINDT=(TIN-TOLD)/DELT

ALPHA=0.5*A*HIA/HOA

TAUC=CM/(HIA+HOA)

ALAG=TSS-TAMB+TAUC*EXP(-A-ALPHA)*DTINDT

C        DTODT=(EXP(-ALPHA)/TAUC)*(TAMB-TOUT+DELAY(ALAG,TAUX,SAVED(1),

C    &    SAVED(7),SAVED(8),SAVED(9)))

DTODT=(EXP(-ALPHA)/TAUC)*(TAMB-TOUT+DELAY(ALAG,TAUX,S(ISAV),

&      S(ISAV+6),S(ISAV+7),S(ISAV+8),INFO))

IF (ABS(DTODT).LT.1.E-6) DTODT=0.0          ! added 2/24/94

OUT(1)=DTODT

C        SAVED(11)=TIN

S(ISAV+10)=TIN

ENDIF

OUT(2)=P1

C     IOSTAT(2)=1

C     IOSTAT(1)=0

C     IF ((ABS(TSS-TOUT).LE.RTOLX*ABS(TSS)+ATOLX).AND.FRZTO)

C    & IOSTAT(1)=1

RETURN 1

END

C***********************************************************************

C     FUNCTION DELAY(TINEW,TAU,A,TIN,DZOLD,TIME0)

FUNCTION DELAY(TINEW,TAU,A,TIN,DZOLD,TIMEZERO,INFO)

C ---------------------------------------------------------------------

C

C     Models transport delays in ducting components.

C     The temperature distribution in the duct is modeled by a fifth

C     order time dependent polynomial. The coefficients of the

C     polynomial are calculated at each time step.

C

C     Modified:  March 5, 1987 C.P.

C

C***********************************************************************

C     PARAMETER (M=5,N=M+1)

C     DIMENSION A(N),AM(M,N)

DIMENSION A(6),AM(5,6),INFO(15)

C     COMMON  /CHRONO/ TIME,TSTEP,TTIME,TMIN,ITIME

C     COMMON  /SOSCOM/ RTOLX,ATOLX,XTOL

C     COMMON  /XINIT/  INIT,NSAVED

COMMON  /SIM/ TIME0,TFINAL,DELT,IWARN

DATA  INIT,M,N /0,5,6/

C     Initialize coefficients if this is the first call.

C     If the transport delay is zero, return the input value.

C     IF (ITIME.EQ.1 .AND. INIT.EQ.0) THEN

IF (INFO(7) .EQ. -1 .AND. INIT .EQ. 0) THEN

TIN=TINEW

A(1)=TIN

C       TIME0=TIME

TIMEZERO=TIME

DELAY=TIN

IF (TAU.GT.0.) THEN

C          DZOLD=TSTEP/TAU

DZOLD=DELT/TAU

ELSE

DZOLD=2.0

ENDIF

DO 10 I=2,N

A(I)=0.

10      CONTINUE

RETURN

ENDIF

C     Set the time step equal to minimum time step if TSTEP < TMIN

C     IF (TSTEP.LT.TMIN) THEN

C        TSTEP=TMIN

C     ENDIF

IF (TAU.GT.0.) THEN

IF (TAU.GT.1.0E+10) THEN

TAU=1.0E+10

ENDIF

C       DZ=TSTEP/TAU

DZ=DELT/TAU

ELSE

DZ=2.0

ENDIF

C     If first call of the timestep, evaluate new coefficients.

C     IF (TIME.GT.TIME0) THEN

IF (TIME .GT. TIMEZERO) THEN

C     Set up matrix and constant vector to solve for new set of

C     coefficients.

IF (DZOLD .LE. 0.5) THEN

DO 30 I=1,M

AM(I,N)=0.

X=1.

XX=1.

IF (I.EQ.1) THEN

IF (DZOLD.GE.0.2) THEN

Z=0.5*DZOLD

ELSE

Z=DZOLD

ENDIF

ELSEIF (I.EQ.2 .AND. DZOLD.GE.0.2) THEN

Z=DZOLD

ELSE

I2=I

Z=0.2*REAL(I2)

ENDIF

DO 20 J=1,M

XX=XX*Z

AM(I,J)=XX

IF (Z.GT.DZOLD) THEN

X=X*(Z-DZOLD)

AM(I,N)=AM(I,N)+A(J+1)*X

ENDIF

20          CONTINUE

IF (Z.LE.DZOLD) THEN

AM(I,N)=(A(1)-TIN)*Z/DZOLD

ELSE

AM(I,N)=AM(I,N)+A(1)-TIN

ENDIF

30        CONTINUE

C     Solve for coefficients by Gaussian elimination.

DO 40 I=2,M

DO 40 J=I,M

R=AM(J,I-1)/AM(I-1,I-1)

DO 40 K=I,N

AM(J,K)=AM(J,K)-R*AM(I-1,K)

40        CONTINUE

DO 50 I=2,M

K=N+1-I

R=AM(K,N)/AM(K,K)

DO 50 J=I,M

L=N-J

AM(L,N)=AM(L,N)-R*AM(L,K)

50        CONTINUE

A(1)=TIN

DO 60 I=1,M

A(I+1)=AM(I,N)/AM(I,I)

60        CONTINUE

C     Last DZ between 0.5 and 1: reset to linear temperature

C     distribution.

ELSEIF (DZOLD .GT. 0.5 .AND. DZOLD .LT. 1.0) THEN

DO 70 I=2,N

A(I)=0.

70        CONTINUE

A(2)=A(1)-TIN

A(1)=TIN

C     Last DZ beyond inlet: reset to uniform temperature.

ELSE

DO 80 I=2,N

A(I)=0.

80        CONTINUE

A(1)=TIN

ENDIF

ENDIF

C     Calculate output.

C-----------------------------------------------------------------------------------------------------------------------!zeug aus type 31 (pipe)

C    DO ALL OF THE INITIAL TIMESTEP MANIPULATIONS HERE - THERE ARE NO ITERATIONS AT THE INTIAL TIME

IF (TIME.LT.(TIME0+DELT/2.)) THEN

C       SET THE UNIT NUMBER FOR FUTURE CALLS

IUNIT=INFO(1)

C       READ IN THE VALUES OF THE PARAMETERS

D=PAR(1)

IF(D.LE.0.) CALL TYPECK(-4,INFO,0,1,0)

L=PAR(2)

IF(L.LE.0.) CALL TYPECK(-4,INFO,0,2,0)

U=PAR(3)

IF(U.LT.0.) CALL TYPECK(-4,INFO,0,3,0)

RHO=PAR(4)

IF(RHO.LE.0.) CALL TYPECK(-4,INFO,0,4,0)

CP=PAR(5)

IF(CP.LE.0.) CALL TYPECK(-4,INFO,0,5,0)

T_INIT=PAR(6)

C       SET THE INITIAL VALUES OF THE OUTPUTS

OUT(1)=T_INIT

OUT(2)=XIN(2)

OUT(3)=0.D0

OUT(4)=0.D0

OUT(5)=0.D0

OUT(6)=T_INIT

OUT(7)=0.

OUT(8)=0.

C       SET THE INITIAL STORAGE VARIABLES

NSTORAGE=4*MAXSEG+2

STORE(1)=1.D0

STORE(2)=1.D0

STORE(3)=T_INIT

STORE(4)=1.D0

STORE(5)=T_INIT

STORE(6)=1.D0

CALL setStorageVars(STORE,NSTORAGE,INFO)

C       RETURN TO THE CALLING PROGRAM

RETURN 1

ENDIF

--------------------------------------------!zeug aus type 31

C     TIME0=TIME

TIMEZERO=TIME

TIN=TINEW

DZOLD=DZ

IF (DZ.GE.1.0) THEN

DELAY=TINEW

ELSE

DELAY=A(1)

X=1.

DO 90 I=2,N

X=X*(1.-DZ)

DELAY=DELAY+A(I)*X

90       CONTINUE

ENDIF

RETURN

END