Question 1

The equation used to get Gw1 in your thesis seems to be different from the one in code.  I think that the codes for calculating Gw1 locate in the constructor of Zone2N in zone.cpp. The discrepancy mainly were found in the following section:

// Resistor calculation

for (unsigned int i=0;i<layersOutside;i++) {

R[i]=0.5*(vLayer->at(i).getxx()/vLayer->at(i).getkw());

for (unsigned int j=0;j < i;j++) {

R[i]+=(vLayer->at(j).getxx()/vLayer->at(j).getkw());

}

}

According to the present codes, take a layerOutside=3 for example, we can get the following R[i] values:

when i=0, R0=0.5d0kw0

when i=1, R1=0.5d1kw1+d0kw0

when i=2, R2=0.5d2kw2+d0kw0+d1kw1

However, according to Eq. (4.11) in your thesis and the relationship R=1/Gw, we can get the following results:

when i=1, −1Gw1+2Gw1=1Gw1=R1=d1kw1 (I think this part should be correspond to  R[0] above )

when i=2, −1Gw2+2Gw1+2Gw2=2Gw1+1Gw2=2R1+R2=2d1kw1+d2kw2 (correspond to R[1])

when i=3, (correspond to R[2])

So by comparisons above, there seem to be some difference between your thesis and codes if I understand correctly. Which one is the right formula for calculating Gw1?

Question 2:

I try to use BESTEST algorithm to validate thermal model in CitySim, but the comparison result is not good enough. I test case 600 first, and produce a xml file for CitySim according to case 600's configuration. You can find the attached case600.xml for detail. The case 600's configuration can be found in Table 1-14(page 1-17) and 1-15 (page 1-18) in BESTEST document which is downloaded from http://www.nrel.gov/docs/legosti/old/6231.pdf .

The reason of selecting such value for some parameters is listed below:

- Vi in xml file is changed to 107.7 from original room's volume 129.6(8m*6m*2.7m) by a factor of 0.822 given in Table 1-4  to account for air density change with the site's altitude(about 1600m). In this case, Ninf is set to 0.5 identical to the value given in Table 1-14 in BESTEST document and air density is still to 1.2 in CitySim code.

- BlindIrradianceCutOff is changed to 2000 to decline the effect of blind.

- Kground for Floor is set to 0.04 according to Table 1-15.

- Uvalue for Roof is set to 0.32 according to Table 1-15.

- GlazingRatio of southern Wall id=203 is set to 0.55556, which is correspond to window area 12 m2.

- GlazingGValue is set to 0.787 which is equal to solar heat gain coefficient given in Table 1-7, and GlazingUValue is set to 3.0 which is equal to U-value from interior air to ambient air given in Table 1-7.

- New WallType with id=39 is produced according to wall type given in Table 1-15.

- The ShortWaveReflectance is set to 0.4 according to 0.6 of  EXT SW ABSORPT as shown in Table 1-14.

- To account for internal heat gains as described in 1.4.4 (page 1-5) in BESTEST document, the source code is modified slightly in about line 80 and line 360 of models.cpp as below:

- Lr=200*0.6;

- Lc=200*0.4;

- The ground temperature is set to 10 according to 1.4.13 section, so source code about Tground in models.cpp is modified slightly as below:

- Tground=10;

- The climate file is originally DRYCOLD.tmy format, which was found from Internet and then was transferred to DRYCOLD.cli by TRNSYS. The original DRYCOLD.tmy is similar to the weather condition of the city Denvor, US. The DRYCOLD.cli file is also attached.

The annual total heating load is  5.898MWh and the annual cooling load is 9.430MWh from CitySim, but the average results provided in BESTEST document (see page 3-2) are that annual heating load is 5.090MWh and annual cooling load is 6.832MWh. The large difference can be seen between CitySim and BESTEST. So I wonder the reasons for this kind of difference.